### CodeAlpha — Robotic Arm Simulation

Task 2: 3D Robotic Arm Simulation with 4-DOF Pick-and-Place Operation

Submitted by: Muhammad Ahmer Khan
Institution: UET-Lahore
Date: 2026-10-03

#### 1. Introduction

This project presents a 3D simulation of a robotic arm with 4 degrees of freedom (DOF) performing a pick-and-place operation. The simulation is developed using Python with NumPy and Matplotlib libraries, demonstrating forward and inverse kinematics, trajectory planning, and gripper control.

The robotic arm is designed to pick an object from one position and place it at another, mimicking industrial automation tasks.

#### 2. Degrees of Freedom (DOF)

The robotic arm has 4 Degrees of Freedom:

| Joint | Name | Length (m) | Rotation Axis | Function |
| --- | --- | --- | --- | --- |
| 1 | Base | 0.5 | Y-axis | Base rotation |
| 2 | Shoulder | 1.0 | Z-axis | Shoulder movement |
| 3 | Elbow | 0.8 | Z-axis | Elbow movement |
| 4 | Wrist | 0.4 | Z-axis | Wrist movement |
| 5 | Gripper | 0.3 | Z-axis | Object gripping |

Note: The gripper is considered an end-effector, not a separate DOF.

#### 3. Forward Kinematics

Forward kinematics calculates the position of each joint and the end-effector using transformation matrices.

Formula:

text

Position = Base + Σ (Rotation Matrix × Joint Direction)

Steps:

- Start at base position (0,0,0)

- Apply rotation matrix for each joint

- Calculate joint direction

- Update position

- Repeat for all joints

Output: End-effector position (x, y, z)

#### 4. Inverse Kinematics

Inverse kinematics calculates the joint angles required to reach a target position.

2-DOF Planar Arm Solution:

text

d = √(x² + y²)

cos(θ₂) = (x² + y² - L₁² - L₂²) / (2 × L₁ × L₂)

θ₂ = arccos(cos(θ₂))

θ₁ = arctan2(y, x) - arctan2(L₂ × sin(θ₂), L₁ + L₂ × cos(θ₂))

Reachability Check:

- If d > L₁ + L₂ → Target unreachable

- If d < |L₁ - L₂| → Target unreachable

#### 5. Pick-and-Place Operation

The pick-and-place cycle consists of 4 phases:

| Phase | Steps | Action |
| --- | --- | --- |
| 1. Move to Pick | 0-50 | Arm moves to pick position |
| 2. Close Gripper | 50 | Gripper closes, object held |
| 3. Move to Place | 50-150 | Arm moves to place position |
| 4. Open Gripper | 150 | Gripper opens, object released |
| 5. Return Home | 150-200 | Arm returns to home position |

Pick Position: (1.5, 1.0, 1.0)
Place Position: (-1.5, 2.0, -1.0)

#### 6. Simulation Features

| Feature | Description |
| --- | --- |
| 3D Visualization | Real-time 3D plot of robotic arm |
| Joint Control | Sliders for manual joint angle control |
| Pick Button | Picks object when gripper is near pick position |
| Place Button | Places object when gripper is near place position |
| Start Demo | Automatic pick-and-place animation |
| Reset | Returns arm to home position |
| Status Display | Shows gripper status, holding status, and DOF |

#### 7. Workspace Analysis

| Axis | Range (m) |
| --- | --- |
| X-axis | -2.5 to 2.5 |
| Y-axis | 0 to 3.5 |
| Z-axis | -2.5 to 2.5 |

Total Workspace: 5m × 3.5m × 5m

#### 8. Performance Metrics

| Metric | Value |
| --- | --- |
| Reachability | Full 3D workspace |
| Precision | Continuous joint angle control |
| Cycle Time | ~10 seconds per pick-and-place |
| Animation | 200 frames per cycle, 50ms interval |

#### 9. Applications

- Industrial Automation: Pick-and-place in factories

- Warehouse Robotics: Sorting and packaging

- Medical Robotics: Surgical assistance

- Education: Teaching robotics and kinematics

- Research: Testing control algorithms

#### 10. Conclusion

The 3D robotic arm simulation successfully demonstrates forward and inverse kinematics, trajectory planning, and pick-and-place operation. The simulation provides a visual and interactive platform for understanding robotic arm control. Future enhancements can include:

- 6-DOF arm for more complex movements

- Obstacle avoidance for real-world scenarios

- AI-based control for autonomous operation

- IoT integration for remote monitoring

Submitted by: Muhammad Ahmer Khan
Institution: UET-Lahore
Date: 2026-10-03